This laboratory assignment accompanies the book, Embedded Systems: Real-Time Interfacing to ARM Cortex M Microcontrollers, ISBN-13: 978-1463590154, by Jonathan W. Valvano, copyright © 2021.

Team Size:	 4

# Goals

- To study ADC conversion and the Nyquist Theorem.

- To characterize the performance of the DAC and ADC.

- To encode information as sound output from DAC to speaker.

- To decode information as sound input from microphone to ADC.

- To develop an audio communication system.

# Review

- Operation of the ADC peripheral in the TM4C123GH6PM data sheet,

- Search http://www.ti.com/ for a data sheet on the TLV5618CP 12-bit DAC (or equivalent DAC),

- Valvano Section 6.2 on periodic interrupts using the timer,

- Valvano Section 7.5 on SSI interfacing,

- Valvano Section 8.4 on DAC parameters and waveform generation.

# Starter files

- Example projects:

  - PeriodicTimer0AInts_4C123 project,

  - ADCT0ATrigger_4C123 project,

  - ST7735_4C123 LCD driver used in Labs 1-5

  - FFT16.xlsx, presented in Lecture aLec49d.pptx

- Starter project: Lab 9 template provided on GH Classroom repo.

  - You will find the TLV5618, the MC34119, and the speaker in the Eagle EE445L library.

# Required Hardware

EK-TM4C123GXL			http://www.ti.com  				$12.99

Sitronix ST7735 Color LCD 	http://www.adafruit.com/products/358 		$19.99

Speaker				Checkout counter

Cell phone Headset	(3.5mm)	Use your own cellphone microphone

Electret Microphone 		Checkout counter

LM4041CILPR shunt diode		From Lab 5

OP2350PA op amp			Handed out already

TLV5618 12-bit DAC		From Lab 5

TPA731 or MC34119, Audio amp	From Lab 5

Various resistors and capacitors	Checkout counter

# Background

This lab introduces the student to the field of audio signal processing. According to Wikipedia:

“Audio signal processing is a subfield of signal processing that is concerned with the electronic manipulation of audio signals. Audio signals are electronic representations of sound waves—longitudinal waves which travel through air, consisting of compressions and rarefactions. The energy contained in audio signals is typically measured in decibels. As audio signals may be represented in either digital or analog format, processing may occur in either domain. Analog processors operate directly on the electrical signal, while digital processors operate mathematically on its digital representation.”

These signals are in the range of 200 Hz to 16000 Hz and include speech and musical instruments. We will be processing the signals in the digital domain which means we need to convert them from the analog domain to the digital domain. In Lecture 39 we covered the Nyquist Theorem which specifies that in order to reproduce an analog signal from a digital signal we need to use greater than a 2X sampling frequency during the analog to digital conversion process. What is the Valvano Postulate?

Figure 9.1 shows the data-flow graph for the audio communication system that you will be building. The human operator provides input data to the system. The input information is first encoded as digital waveforms. The waveforms are output to the DAC/amp/speaker to create sounds. The sounds transmit across the air as pressure waves. The electret/amp/ADC converts the sounds into a sequence of data. The decoder converts the data back to the information (hopefully). Lastly, the output displays that information to the human operator.

Figure 9.1.   Data-flow graph of the audio communication system

- Input: The input to the system can arrive whatever source you wish, as long as the human operator controls the values in some fashion. You could input characters from the PC keyboard and input them into the microcontroller using UART. You could input data from switches. The input task must run in the background using interrupts. Think about the appropriate priority level for this task.

- Input → Encoder: The linkage between these modules must include a data structure that supports streaming, such as a FIFO or double buffer. A level of abstraction must be created by designing a set (two or more) of public functions the input module can call to affect communication.

- Encoder: Each communication effort will involve a message, which includes data, synchronization, and error checking. You are free to create whatever encoding mechanism you wish. However, consider encoding each n-bit symbol as a specific frequency output for a fixed time (frequency modulation). One of the challenges will be synchronize the receiver to the transmitter. Even if the receiver knows you are sending a symbol encoding 2 bits of data every 10 ms, how does the receiver know where symbol ends and the next symbol starts? Consider a protocol where subsequent symbols always switch frequencies, even if the data were to remain constant. In Lab 5, recall that you set up a free running sine wave routine that can generate a wide range of sounds from 300 Hz to 1000 Hz. You will use that capability to encode data using sine waves (300 Hz and 1000 Hz). There are no particular requirements to maximize bandwidth (information/sec), minimize latency (time from input to output), or eliminate errors (sound noise causing bits to flip). However, you will be asked to quantify each of these performance measures and discuss in your report how each could have been improved. The encoder task also runs in the background using interrupts. Think about the appropriate priority level for this task. Remember there is a streaming data structure, so this interrupt task is different from the input task.

- Encoder → DAC: The encoded message is sent to the DAC module in an appropriate manner that you will design. For example it might be an array of frequencies. The linkage between these modules also must include a data structure that supports streaming. However, you may wish to place all transmission tasks (encoder and DAC) into the same software file.

- DAC: Please use as much of your Lab5 as possible because the DAC does output to the speaker to send data across the communication channel. Since this interrupt will be high frequency, make sure it is very simple. Think about the appropriate priority level for this task.

- DAC → ADC: There must be a software disconnect between the transmitter tasks (input, encoder, and DAC) from the receiver tasks (ADC, decode, display). There can be no shared data or function calls between them. The only linkage should be sound traveling as air pressure.

- ADC: The audio input is filtered and then sampled by an ADC. The sampling rate is controlled by a periodic timer that triggers the ADC, and the ADC ISR executes when the conversion is complete. Because the timer starts the ADC, there is no sampling jitter. Think about the appropriate priority level for this task. How does the timer-triggered sampling affect the selection of priority for this ISR? Since this interrupt will also be high frequency, make sure it is very simple. However, you should implement digital filtering here to improve SNR. See some of the filters provided in the sw/inc folder of the GH classroom repo.

- ADC → Decoder: The linkage between these modules also must include a data structure that supports streaming.

- Decoder: You can run the decoder in the while-loop of main. The goal is to extract the data from the sampled sound. The system is considered real time if the software is fast enough to keep up (no data is lost). If the decoder cannot keep, up you can slow down the transmission rate or use a simpler decoding algorithm. Possibilities include but are not limited to: cross correlation and FFT. There are three very fast integer FFT functions in the inc folder of the starter projects: cr4_fft_64_stm32.s, cr4_fft_256_stm32.s cr4_fft_1024_stm32.s.

- Decoder → Display: The linkage between these modules can be a simple function call.

- Display: This task is run from the main program. It outputs the data to the human operator. The output can use whatever device you wish, as long as the human operator obverse the values in some fashion. You could output characters on the LCD. You could output data to LEDs. During testing, when the input is created with data of known values, the output can check for lost or changed values.

The max sampling rate on the TM4C123 is 125K samples/sec which is way above the Nyquist value (and the Valvano Postulate) for audio input signals. You should select sampling rates that allow all tasks to complete in real time.

All I/O must be written in a style like the book, without calling any TivaWare driver code. It is acceptable but not necessary to use two microcontrollers, one for transmitting and the other for receiving.

Figure 9.2 shows possible a possible dataflow graph for the audio communication system that you will be building. Feel free to change how the pieces fit together as long as the data streams and there is no wasted software delays.

Figure 9.2.  Possible dataflow graph of the communication system and relevant questions to consider.

# Preparation (do this before your lab period)

- Choose an encoding/decoding scheme. Make some rough sketches of the sound waves you plan to use to transmit data. You are free to change this as you build it and see what works and what does not work.

- Choose a message format. Add something to the protocol to support error checking (like parity or checksum). Add something to the protocol that will allow the receiver to synchronize to the transmitter. How will the receiver be able to separate one bit from another? Hint, how does the asynchronous UART protocol separate bits? Be able to provide an explanation of your message format.

- Distribute tasks. Provide a list of tasks of software to write, hardware to build, and tests to perform, equally distributed between team members. Ideally, everyone is working on something in parallel.

- Hardware Design. Collect all components you may use and provide a schematic of the encoding and decoding systems. You may use a headset microphone and an audio jack connector in place of an electret microphone. A circuit for the electret microphone is provided for you in Figure 9.4.

Figure 9.3 Audio Signal Input Circuit

Figure 9.4 Possible Electret Circuit

- Software Design. Design header files for software modules and data structures. Write unit tests, potentially for each block in your call graph, demonstrating that each component works in isolation. You may then want to write integration tests to check that interactions between components work as well. The final main will likely be the combination of your integration tests.

# Procedure (do during lab period)

Note: We assume that you have implemented and tested the modules separately. These are the test and performance validation procedures to be executed assuming that the system is implemented and works (relatively).

- Encoder

  - Connect a scope to the DAC output.

  - Tweak output parameters (sine wave table size, encoding scheme, symbol rate) of your encoder until the output waveforms for an arbitrary input meet the requirements of your designed message protocol on the scope.

  - Capture the scope trace and put this in your lab report along with the necessary context (what was the input, sketch of what the output looks like compared to actual, some timing info, etc).

- Decoder

  - Verify that the decoder can decode the above encoder output stream correctly under controlled test conditions.

  - Capture the ADC input from the output stream and dump it to a spreadsheet or visualization software. Show this in the lab report (the quantized waveform, noise and all). Provide context on the test setup, including but not limited to:

    - Is it quiet? Is it loud?

    - What speaker did you use? What microphone did you use?

    - How far away were the input and output from each other?

  - Profile the system modules’ relative percentage execution time with a logic analyzer.

  - Quantify the system performance:

    - Bandwidth

    - Latency

    - Reliability

# Demo

- Show that the system works with an arbitrary input, with DAC scope output and ADC quantized input plot (see Procedure).

- Demonstrate the system works with user input.

- Demonstrate that the system is robust (or best attempt robust) under noisy environments.

- Explain the encoding/decoding scheme.

- Explain any synchronization or robustness schemes implemented.

# Deliverables (exact components of the lab report)

- A summary of the lab and what the student expects to learn from it.

- A summary of the software and hardware that was created to complete the lab (keep the full software on GH, paste a screenshot/printout of HW in the report).

- Measurements as specified in the procedure.

- Analysis and Discussion

  - What is the Nyquist theorem and how does it apply to this lab?

  - How did you eliminate noise in the sampled audio?

  - How does your protocol allow (or doesn’t allow) communication in the presence of background speech and singing?

  - How could bandwidth have been improved?

# Hints (Example waveforms)

The following figure is the analog signal at the DAC output showing a piece of an FM-modulated communication. This was measured running just the transmitter with TExaS initialized with the SCOPE_PD2 setting. The higher frequency encodes a zero bit and the lower frequency encodes a one bit. This symbol time is 10.24ms.

Figure 9.4 FM-modulated encoding of Audio Signal

The following is a similar waveform of DAC out measured on a real scope. This format is start bit (0), 8 data bits (10101010), and even parity (0).

Figure 9.5 FM-modulated encoding of Audio Signal, one frame

The following figure is the analog signal at the input of the receiver ADC showing a piece of an FM-modulated communication. This was measured running just the transmitter with TExaS initialized with the SCOPE_PE2 setting. The higher frequency encodes a zero bit and the lower frequency encodes a one bit. The symbol time is 10.24ms.

Figure 9.6 FM-modulated decoding of Audio Signal, some bits

The following figure is the analog signal at the input of the receiver ADC showing the entire FM-modulated message, measured with a real scope. The time to transmit one message is 102.4ms.

Figure 9.7 FM-modulated decoding of Audio Signal, one frame

Let fs be the receiver sampling rate. DFT length will be N=16

Let f1 be one frequency, which will be equal to fs /16

Let f2 be the second frequency, which will be equal to 2* fs /16

See the functions in dsp.c dsp.h

https://www.dropbox.com/s/tldoeap4h1oyynz/dsp.c?dl=1

https://www.dropbox.com/s/w62j5dn7p6jzd2d/dsp.h?dl=1

Usage for decoding FM modulation in receiver
- Call DFT_Init() once
- at fs in real time using periodic interrupt
   1) x=sample microphone using ADC
   2) call DFT(i,x) once per interrupt, with index i going from 0 to 15
   3)  after 16 samples
       a) call Mag1() for relative amplitude at f1
       b) call Mag2() for relative amplitude at f2

The Discrete Fourier Transform (DFT) converts data in the time domain to data in the frequency domain. We can use the DFT to measure SNR, to identify noise type, and to design FIR digital filters. In fact, the spectrum analyzer is simply a high-speed data acquisition system followed by a DFT. The Fast Fourier Transform (FFT) is a technique to calculate the DFT with fewer additions and multiplications. There are four important parameters when employing the DFT. The first parameter is sampling rate, fs. While the DFT deals only with samples and bins with no concept of volts, seconds, and Hz, when applying it to real data, we assume the samples have units, are bound by physical limits, and are evenly spaced at time intervals T=1/fs. The second parameter is sequence length, N. The other two parameters are input resolution and range. In real systems, input data come from the ADC or input capture, and the output data go to the DAC or PWM. Therefore, the performance of the DFT will be affected by the range and resolution of the input. The input to the DFT will be N samples versus time, and the output will be N points in the frequency domain.

Input: 	{an} = {a0,a1,a2,…,aN-1}

Output: 	{Ak} = {A0,A1,A2,…,AN-1}

The definition of the DFT is

where  		and  k=0,1,2,…,N-1

The DFT output Ak at index k represents the amplitude and phase of the input at frequency k*fS/N (in Hz). The DFT resolution in Hz/bin is the reciprocal of the total time spent gathering time samples; i.e., 1/(N*T).